Notes

Retuned for the locked config (4T4R, 200 MHz iBW, TDD, shared/reduced DPD) and updated the document. Headline result:

The design fits a single ZU48DR, but LUT/routing is the binding resource — estimated ~250k–400k LUTs (~59–94%), while DSP (~1,800–3,000, ~42–70%), FF, BRAM and UltraRAM all stay below ~70%.

What each of your choices did to the numbers: 200 MHz iBW roughly doubles the datapath rate versus a 100 MHz design and is what pushes LUT to the front; TDD lets TX/RX time-share and the DPD observation path reuse the RX front-end, trimming fabric versus FDD; and the shared/reduced DPD is the main reason it fits at all — full per-antenna DPD at 200 MHz would likely overflow the device.

If LUT gets tight in synthesis, the levers are fewer component carriers, a lower DPD model order, or a -2 speed grade for timing headroom. These are still first-order estimates — a trial synthesis on the actual RTL is what confirms them.

In the doc, the config-fit table now carries your target row at the top, and Section 7's utilization subsection is rewritten for this exact config with the per-resource table and the modeling notes.

One caveat on the CC count: I assumed up to ~4 component carriers packed within the 200 MHz. If your carrier plan is different (e.g., a single 200 MHz NR carrier vs. several narrower ones), tell me and I'll adjust the filter/mixer replication accordingly.