AMD (Xilinx) RFSoC DFE

Device features, hard/soft core inventory, and feasibility of a Gen-3 non-DFE implementation

Technical brief

# 1. What the RFSoC DFE is

The Zynq UltraScale+ RFSoC DFE is a purpose-built O-RU silicon line (ZU63DR / ZU64DR / ZU65DR / ZU67DR; ZCU670 evaluation board) that takes the Gen-3 RF-sampling data converters and adds a hardened digital-front-end (DFE) signal chain. The compute-heavy DFE math runs in ASIC-like hard IP rather than in fabric. The hardened DPD and CFR blocks are derived from AMD's proven soft cores and enhanced to meet 400 MHz instantaneous bandwidth; the direct-sampling converters are based on Gen-3 with enhancements to operate up to 7.125 GHz. The net result is roughly 2x performance/watt versus Zynq UltraScale+ RFSoC Gen 3.

Headline specifications. Single-band, multi-band and multi-mode LTE and 5G NR; 400 MHz iBW in FR1 (up to 7.125 GHz) and up to 1600 MHz in FR2; up to 8 component carriers for 8T8R FDD and 16 in TDD; integrated feedback ADCs for closed-loop DPD. Notably, SD-FEC is removed (0 on all DFE parts) because forward error correction lives in the DU, not the O-RU.

# 2. Hard vs. soft core inventory

AMD's product selection guide splits the silicon into two hardened categories plus everything still built in the programmable logic (PL):

| Category | Blocks | Hard or soft |
| --- | --- | --- |
| RF Data Converter | 14-bit RF-ADC w/ embedded DDC, 14-bit RF-DAC w/ embedded DUC, NCO/mixer, decimation/interpolation | Hard |
| DFE Hard IP | Channel Filter, DUC/DDC, Mixer, CFR, Complex Equalizer, PQ Resampler, DPD | Hard |
| Low-PHY Hard IP | FFT/iFFT, PRACH (present on ZU63/65/67DR; None on ZU64DR) | Hard |
| Processing System | Quad Cortex-A53 + dual Cortex-R5F | Hard |
| Fronthaul / transport | eCPRI / O-RAN 7.2x FHI, C/U/S/M-plane, transport, BFP compression, RE mapping, digital beamforming, 1588/SyncE, GTY transceivers, custom logic | Soft (PL) |

Each hard block ships as a LogiCORE with its own product guide, e.g. PG390 (FFT), PG391 (PRACH), PG394 (Equalizer), PG395 (Channel Filter).

## Two clarifications against the common assumptions

- FFT/iFFT and PRACH are hardened in the DFE (as Low-PHY hard IP). The exception is the ZU64DR, the split-8 8T8R FDD part, where the low-PHY sits in the BBU, so those blocks are absent.

- eCPRI and O-RAN are not hardened. The fronthaul stack (eCPRI/O-RAN 7.2x fronthaul interface, C/U/S/M-plane, transport, compression, beamforming, RE mapping and sync) is soft IP in the PL, from AMD's O-RAN reference design plus partners (RF DSP Inc., iCana). The ZCU670 reference design integrates FHI, O-RAN processing and Low-PHY (split 7.2) in the fabric alongside the DFE processing.

CFR detail. The CFR hard IP provides three stages of pulse-cancellation CFR (PC-CFR), limited to 400 MHz iBW from the ~491.52 MSPS hard-IP clock. For wider signals (FR2) a Window CFR (W-CFR) stage is included, which handles wider bandwidth but with lower performance and some spectral spreading.

# 3. Feasibility on a Gen-3 non-DFE RFSoC

Implementing the DFE functions on a Gen-3 non-DFE RFSoC is feasible; it is in fact how O-RUs were built before the DFE part existed. The natural comparison target is the XCZU48DR (Gen-3, 8x8 converters), the closest non-DFE analog to the 8T8R ZU67DR.

AMD's own guidance is explicit: the ZU67DR contains not only the embedded RF Data Converter but also several hard cores implementing DUC/DDC, Resampler, PRACH, DPD and so on; the ZU48DR does not have those hard cores, and it is up to the customer to build them in logic/DSP.

## Porting map

- Ports directly (already soft on both): the entire eCPRI/O-RAN fronthaul, transport, beamforming, compression, RE mapping and sync. No rework, this was never hardened.

- Free on Gen-3 via the RFdc hard block: coarse per-tile DDC/DUC, NCO mixing and decimation/interpolation.

- Must be rebuilt in soft logic/DSP: per-carrier Channel Filter, fine Mixer, CFR, DPD (with feedback-ADC adaptation), Complex Equalizer, PQ Resampler, and the Low-PHY FFT/iFFT + PRACH.

AMD historically shipped soft CFR/DPD/DUC-DDC IP (the same cores the DFE hard blocks were derived from) and third-party DFE IP is available, so not all of it is written from scratch.

# 4. Signal-chain partitioning

The diagram and table below classify each stage of the transmit and receive chains: what the DFE hardens (rebuild in PL on Gen-3), what is soft on both parts (ports unchanged), and what comes from the Gen-3 RFdc hard block (available on Gen-3).

Figure 1. TX/RX signal-chain partitioning for a Gen-3 (ZU48DR) soft-IP O-RU.

| Chain | Stage | Classification on Gen-3 |
| --- | --- | --- |
| Transmit (DL) | Fronthaul | Soft on both — ports unchanged |
| Transmit (DL) | iFFT / CP insertion | DFE hard IP — rebuild in PL |
| Transmit (DL) | DUC + channel filter | DFE hard IP — rebuild in PL |
| Transmit (DL) | Fine mixer | DFE hard IP — rebuild in PL |
| Transmit (DL) | CFR | DFE hard IP — rebuild in PL |
| Transmit (DL) | DPD | DFE hard IP — rebuild in PL |
| Transmit (DL) | RF-DAC | RFdc hard block — available on Gen-3 |
| Receive (UL) | RF-ADC | RFdc hard block — available on Gen-3 |
| Receive (UL) | DDC + channel filter | DFE hard IP — rebuild in PL (coarse DDC free) |
| Receive (UL) | Complex equalizer | DFE hard IP — rebuild in PL |
| Receive (UL) | FFT / PRACH | DFE hard IP — rebuild in PL |
| Receive (UL) | Fronthaul | Soft on both — ports unchanged |

# 5. Hardened stage to soft-IP replacement

| DFE hard stage | Soft-IP equivalent on Gen-3 | Notes |
| --- | --- | --- |
| DUC/DDC (coarse) | RFdc hard block (NCO + interp/decim) | Free on Gen-3 — no rebuild |
| Channel filter | LogiCORE FIR Compiler chain (per CC) | Per-carrier filtering + fine rate change |
| Mixer (fine) | DDS Compiler + complex multiply | Per-CC NCO for carrier placement |
| CFR | AMD/partner soft CFR (PC-CFR) or RF DSP Inc. IP | Multi-stage peak cancellation |
| DPD | AMD/partner soft DPD + adaptation in PS/PL | Dominant cost; needs observation-Rx capture |
| Complex equalizer | Soft complex FIR / IQ-imbalance correction | Straightforward |
| PQ resampler | Farrow / polyphase resampler | Straightforward |
| FFT/iFFT (low-PHY) | LogiCORE FFT | 5G NR up to 4096-pt, multiple instances |
| PRACH | FFT-based soft PRACH | Modest |
| eCPRI/O-RAN FHI, BFP, beamforming, 1588/SyncE | Same soft IP as on the DFE part | No change — never was hardened |

# 6. Exact IP cores to instantiate (Gen-3 solution)

These are the soft LogiCORE IP blocks you would instantiate in the PL on a ZU48DR to rebuild the DFE chain. Product-guide numbers link to the AMD documentation.

| Block / function | Soft LogiCORE IP | Product guide |
| --- | --- | --- |
| RF converters, coarse DDC/DUC, NCO | Zynq UltraScale+ RFSoC RF Data Converter | PG269 |
| Per-carrier digital up/down conversion | DUC/DDC Compiler | PG147 |
| Channel / pulse-shaping / resampling filters | FIR Compiler | PG149 |
| High-ratio decimation / interpolation | CIC Compiler | PG140 |
| Fine NCO / carrier mixing | DDS Compiler | PG141 |
| Complex mixing / IQ multiply | Complex Multiplier | PG104 |
| Low-PHY iFFT/FFT and FFT-based PRACH | Fast Fourier Transform | PG109 |
| CFR | Third-party (e.g. RF DSP Inc.) or custom | no public AMD soft core |
| DPD (+ adaptation) | Third-party (e.g. RF DSP Inc.) or custom | no public AMD soft core |
| eCPRI / O-RAN 7.2x fronthaul, Low-PHY, BFP | AMD O-RAN reference design + partner IP | reference design |

Hardened equivalents for reference (DFE part only). On the RFSoC DFE these functions are the hardened LogiCOREs: Channel Filter (PG395), DUC/DDC Mixer (PG393), Equalizer (PG394), FFT (PG390), PRACH (PG391), OFDM (PG434). These target the DFE hard IP and are not used on a Gen-3 device; they are listed only to map soft cores to their hardened counterparts.

# 7. Resource and power budget (ZU48DR)

Assumptions: single-device FR1 O-RU, split 7.2x, closed-loop DPD with observation receiver, datapath at the ~491.52 MSPS hard-IP-class clock, DDC/DUC coarse stages taken from the Gen-3 RFdc hard block, everything else rebuilt in the PL.

Target device XCZU48DR: ~930K logic cells, ~425K LUTs, ~850K FFs, 4,272 DSP48E2, 1,080 BRAM (38 Mb), 80 UltraRAM (22.5 Mb), 16 GTY; 8x 14-bit ADC (up to ~5 GSPS), 8x 14-bit DAC (up to ~9.85 GSPS).

Estimate note: the figures in this section are order-of-magnitude engineering planning estimates, not vendor datasheet numbers. DPD and CFR costs vary widely with algorithm, memory depth and oversampling factor. Treat them as ranges for feasibility scoping, then confirm against synthesis for a specific configuration.

## Per-antenna DFE cost (TX + RX)

| Function (per antenna, TX+RX) | DSP48E2 (approx.) |
| --- | --- |
| Channel filters + DUC/DDC fine + mixer | 80–180 |
| CFR (3-stage PC) | 50–120 |
| DPD actuator (wideband, memory polynomial / GMP) | 120–320 |
| Equalizer + resampler | 40–100 |
| Per-antenna subtotal | ~290–720 |

Shared low-PHY/fronthaul (FFT/iFFT x N, PRACH, BFP compression, beamforming, transport) adds roughly 200–600 DSPs plus heavy BRAM/UltraRAM.

## Configuration fit against 4,272 DSP48E2

| Config | Est. DSP demand | Fit on ZU48DR |
| --- | --- | --- |
| 4T4R, 200 MHz iBW, TDD, shared DPD (target) | ~1,800–3,000 | Fits — LUT-bound |
| 4T4R, 100 MHz iBW, full DPD | ~1,400–2,600 | Comfortable |
| 4T4R, 400 MHz iBW, full DPD | ~2,600–4,000 | Tight but feasible |
| 8T8R, 100 MHz iBW, full DPD | ~2,800–4,400 | Tight; may need DPD sharing |
| 8T8R, 400 MHz iBW, full DPD | ~4,500–6,500 | Does not fit one device |

## Full logic-utilization estimate — target config (4T4R / 200 MHz / TDD / shared DPD)

Target configuration: 4 antennas (4T4R), 200 MHz iBW per antenna, TDD, shared/reduced DPD (one coefficient-adaptation engine time-shared across the four antennas, per-antenna actuators retained at reduced model order); up to ~4 component carriers assumed within the 200 MHz. Percentages are of the ZU48DR fabric.

- 200 MHz iBW roughly doubles the datapath sample rate versus a 100 MHz design, so channel-filter, CFR and DPD-actuator cost scales up ~2x — this is what pushes LUT to the front as the binding resource.

- TDD lets the TX and RX datapaths time-share and the DPD observation path reuse the RX front-end, trimming fabric versus an equivalent FDD design.

- Shared / reduced DPD keeps per-antenna actuators but shares the coefficient-adaptation engine across all four antennas, cutting the dominant DSP term relative to full per-antenna DPD — the main reason the design still fits.

| Resource | ZU48DR available | Est. used | % of device | First to bind? |
| --- | --- | --- | --- | --- |
| LUT | 425,280 | ~250k–400k | ~59–94% | Yes — binding resource |
| Flip-flops | 850,560 | ~350k–560k | ~41–66% | — |
| DSP48E2 | 4,272 | ~1,800–3,000 | ~42–70% | — |
| BRAM (36 Kb) | 1,080 | ~180–380 | ~17–35% | — |
| UltraRAM | 80 | ~20–48 | ~25–60% | Watch DPD / fronthaul buffers |

Reading the estimate. At 4T4R / 200 MHz / TDD with shared DPD the design fits a single ZU48DR, but LUT and routing are the binding resource: a heavy CFR/DPD build can approach the device LUT limit, making floorplanning and timing closure at the sample clock the real risk. DSP, BRAM and UltraRAM all sit below ~70%. If LUT gets tight, the levers are fewer component carriers, a lower DPD model order, or a -2 speed grade for timing headroom. Full per-antenna DPD at this iBW would likely overflow the device — the shared DPD scheme is doing the heavy lifting. These remain first-order numbers; a trial synthesis on the actual RTL is needed to confirm.

# 8. Limitations and degradation vs. the DFE part

- Power. The hardened chain is the source of the ~2x perf/watt advantage. A soft 8T8R chain realistically adds on the order of tens of watts of PL dynamic power versus the hardened equivalent, often the binding constraint for a passively cooled RU before logic runs out.

- Resource fit and channel count. The soft DFE chain (especially wideband DPD and channel filters) is very DSP/LUT-hungry; you may have to de-scope antennas, iBW or component carriers, or move to a larger, costlier device.

- DPD quality. The DFE DPD is tuned for wideband GaN PAs. A soft DPD typically offers narrower correction bandwidth and weaker adaptation, giving worse ACLR/EVM and therefore lower PA efficiency, which compounds the power problem at the amplifier.

- RF reach. Gen-3 converters top out near 6 GHz versus the DFE's 7.125 GHz, so upper FR1 bands (n104, 6425–7125 MHz) may be out of reach.

- Timing and latency. Closing timing on wide soft DPD/CFR/FFT at ~491.52 MSPS-class rates usually forces more parallelism (more resources) or a lower clock, and soft datapaths add latency that must still fit the O-RAN budget.

- NRE, time-to-market and BOM. You give up pre-verified, characterized hard blocks and take on the verification burden yourself, on a device that is larger, hotter and more expensive, precisely the ASIC-like cost/power advantage the DFE part was created to deliver.

# 9. Bottom line

Functionally the DFE chain is portable to a Gen-3 non-DFE RFSoC, and it is a well-trodden path. Expect meaningful power and resource penalties, a likely reduced antenna/bandwidth envelope for a given device, and softer DPD/CFR performance. For a high-volume FR1 O-RU where power and cost dominate, the DFE part is hard to beat. For lower-volume designs, unusual configurations the hard IP does not support, or when a Gen-3 platform and soft DFE IP are already in hand, the Gen-3 route is reasonable.

To tighten the budget: fixing the exact target (antenna count, iBW, TDD/FDD, CC count, and whether DPD must be per-antenna or can be shared) allows the ranges above to collapse to a single-config estimate and identifies the specific blocks most at risk of not closing timing.