Split 7.2 with non-DFE solution

The AMD/Xilinx Zynq UltraScale+ RFSoC DFE (Digital Front-End) represents a shift from general-purpose FPGA fabrics toward an application-specific architecture tailored for 5G NR and 4G cellular radio units (O-RAN / macrocells / massive MIMO). Unlike earlier generations (Gen 1–3), which implemented most digital radio processing using soft fabric and DSP slices, the DFE device integrates hardened ASIC-class radio blocks alongside programmable logic.

https://www.scribd.com/document/713295010/Zynq-RFSoC-DFE-ZCU670-Evaluation-Kit-Product-Brief#:~:text=5G%20New%20Radio%20(5G%20NR)%20%3E%208x,DFE%20configuration%20and%20control.%20Aerospace%20and%20Defense.

https://6gspacelab.uni.lu/hardware-facilities/fpga-boards#:~:text=*%20Lidless%20package%20for%20improved%20thermal%20dissipation.,cage%20assembly.%20*%208%20user%2DI%2FO%2C%20single%2Dcolor%20LEDs.

| Subsystem | Key Capabilities |
| --- | --- |
| Hardened DFE IP | Dedicated ASIC blocks for DPD, CFR, DUC/DDC, and Low-PHY filtering, delivering ~2x performance/watt over soft-logic implementations. |
| Direct RF Converters | Multi-GSPS RF-ADCs and RF-DACs covering up to 7.125 GHz RF input/output frequencies without analog IF stages. |
| Programmable Logic (PL) | UltraScale+ FPGA fabric (LUTs, DSP48E2 slices, UltraRAM/BRAM) for custom algorithms, beamforming, and standard evolution. |
| Processing System (PS) | Quad-core Arm Cortex-A53 (APU) and dual-core Arm Cortex-R5F (RPU) for Linux control stacks, calibration, and real-time management. |
| Integrated Connectivity | High-speed transceivers (up to ~28–32 Gbps) supporting eCPRI, CPRI, and 100G/25G Ethernet direct-to-radio. |

Core Functional Features

1. Hardened 5G NR Radio Processing Chain

- Digital Pre-Distortion (DPD): Hardened DPD engines optimized for wideband multi-carrier signals and modern GaN power amplifiers, saving millions of gates of soft logic.

- Crest Factor Reduction (CFR): Peak-to-average power ratio (PAPR) reduction blocks using techniques like Peak Cancellation (PC-CFR).

- DUC & DDC: Digital Up-Conversion and Down-Conversion engines with wide programmable decimation/interpolation filters, mixers, and numerical control oscillators (NCOs) supporting multi-band configurations.

- Low-PHY / Channel Filtering: Hardened channelization filters for 3GPP channel bandwidths (up to 400 MHz instantaneous bandwidth).

- Bypass Capability: Every hardened block can be partially or completely bypassed into the FPGA fabric, allowing custom proprietary algorithms where differentiation is needed.

2. High-Frequency Direct RF Sampling

- Eliminates discrete mixers, local oscillators, and bulky external JESD204B/C serial links between data converters and FPGA fabric.

- Operates natively across Sub-6 GHz and C-band (up to 7.125 GHz), providing support for 8T8R multi-band configurations and dedicated feedback/observation receivers for DPD adaptation.

3. Power & Footprint Optimization

- By replacing heavily utilized FPGA lookup tables and DSP slices with hardened silicon for standard-defined operations, the DFE device slashes dynamic power consumption by up to 50% compared to pure soft-logic implementations in Gen 3 devices.

- Allows massive MIMO (e.g., 32T32R, 64T64R) and high-power macro cell radio heads to fit strict thermal, weight, and mast-mounting constraints.

You're spot on—the O-RAN Split 7.2x partition and the hardened FFT/IFFT engines are core reasons the RFSoC DFE was built in the first place.

In standard 5G Open RAN, the functional split places the boundary between the Distributed Unit (O-DU) and the Radio Unit (O-RU) right across the frequency-to-time boundary. The DFE architecture was optimized explicitly around this split.

Hardened FFT / IFFT Subsystem

In previous generations (Gen 1–3), running multi-carrier, multi-channel FFT/IFFT in soft FPGA fabric devoured substantial DSP slices, block RAM, and dynamic power. The RFSoC DFE hardens these transforms directly into silicon:

- Direct Support for 3GPP Numerologies: Native hardware handling of scalable OFDM subcarrier spacings (μ=0,1,2 / 15 kHz, 30 kHz, 60 kHz) and transform sizes from 512 up to 4096 points.

- Integrated CP Handling: Cyclic Prefix (CP) addition on the Tx side and CP removal/windowing on the Rx side are integrated directly alongside the transform engines.

- Multi-Carrier / Multi-Sector Capacity: Processes up to 800 MHz of aggregate bandwidth or multiple spatial streams simultaneously without touching the programmable logic fabric.

- Low-Latency Pipelining: Direct streaming interfaces route symbols directly from the frequency domain to time domain filtering (DUC/CFR/DPD) with deterministic hardware latency.

O-RAN Split 7.2x Implementation

The device provides a turnkey, single-chip pipeline mapped to the O-RAN 7.2x Category A and Category B functional split:

- Fronthaul Termination: GTY/GTYP transceivers interface directly with 10G/25G/100G optical links to ingest eCPRI/RoE frames from the O-DU.

- eCPRI & O-RAN Protocol Layer: Transport protocol parsing, User-Plane (U-Plane), and Control-Plane (C-Plane) processing are typically handled in an optimized mix of low-footprint soft IP cores (LogiCORE O-RAN Radio Interface) and the ARM Cortex cores (M-Plane and S-Plane IEEE 1588 / SyncE alignment).

- IQ Decompression & Modulation: Unpacks compressed frequency-domain resource elements (e.g., BFP - Block Floating Point 9-to-14 bit) coming over eCPRI.

- Direct Hand-off to Hard IP: Once uncompressed, the frequency-domain symbols pass straight into the hardened IFFT block, proceed through hardened CFR and DPD, and feed directly into the multi-GSPS RF-DACs—minimizing routing congestion across the FPGA interconnect.

- Beamforming: Depending on whether it is a Cat A or Cat B implementation, digital beamforming weights are applied either prior to or alongside the frequency-domain processing blocks in the programmable fabric.

is it feasible to implement the DFE units on a gen-3 RFSoC non-DFE device? which units are moe likly to fit. consider reduced perfomance as possible option

es, it is entirely feasible to implement the DFE signal chain in soft fabric on a Gen-3 RFSoC (e.g., ZU47DR, ZU48DR, ZU49DR). In fact, prior to the introduction of the RFSoC DFE, cellular base stations and RRUs routinely ran these functions in soft logic.

However, scaling is the bottleneck. While a Gen-3 part can handle a 2T2R or 4T4R radio with reduced instantaneous bandwidth (IBW), implementing a full 8T8R multi-carrier 400 MHz chain will exhaust the device’s DSP slices, routing channels, and thermal headroom.

Likelihood of Fitting: Block Breakdown

| DFE Function | Fit Feasibility | Resource Pressure | Trade-offs & Degradation Options |
| --- | --- | --- | --- |
| FFT / IFFT | High | Moderate (DSP48E2 + BRAM) | Easy fit using LogiCORE FFT IP. You can scale down to radix-4/radix-2 pipelined architectures. Easily supports standard 2048/4096-pt sizes for 2T2R/4T4R without overwhelming the fabric. |
| DUC / DDC | High | Moderate (DSP + LUT) | Gen-3 RF-DACs and RF-ADCs already include hard 1x–8x interpolation/decimation filters and mixers. Only fine channel filtering or multi-carrier aggregation needs to go in soft logic. |
| CFR (Peak Cancellation) | Moderate | Moderate (BRAM + LUT) | Highly feasible if the number of iterations and cancellation stages is limited (e.g., 2–3 stages instead of 5+). Trades off 1–2 dB of Peak-to-Average Power Ratio (PAPR) reduction. |
| DPD (Pre-Distortion Engine) | Low to Moderate | Very High (DSP + BRAM/URAM) | Consumes significant DSP slices per antenna path. To fit, you must prune polynomial terms (Volterra series/GMP), lower the expansion order, reduce observation bandwidth, or update model weights less frequently. |
| eCPRI / O-RAN Deframer | High | Low (LUT + Transceivers) | Fits readily into logic using AMD/Xilinx LogiCORE O-RAN Radio Interface or custom framing RTL. |

What Can Realistically Fit on a Gen-3 Device?

1. A Capable 2T2R or 4T4R Narrower-Band Chain (Highly Realistic)

- Bandwidth: 100 MHz to 200 MHz IBW.

- Architecture: Soft FFT/IFFT + 2-stage Peak Cancellation CFR (PC-CFR) + a simplified Generalized Memory Polynomial (GMP) DPD.

- Utilization: A large Gen-3 device like the ZU48DR/49DR (~4,272 DSP slices, ~425k LUTs) will sit around 60%–75% DSP utilization, leaving enough margin for timing closure and clock routing at 300–400 MHz.

2. A Full 8T8R or Massive MIMO Macro Chain (Not Feasible)

- An 8T8R 5G NR configuration with 400 MHz IBW requires massive oversampling for DPD (linearizing a 400 MHz signal up to the 5th order requires processing ~2 GHz of bandwidth in the feedback path).

- In soft logic, this alone can exceed 6,000–8,000 DSP slices—well beyond what any single Gen-3 FPGA die provides.

Practical Trade-Offs to Fit on Gen-3

- Reduce DPD Complexity: Cut high-order cross-memory terms. This costs 2–4 dB in Adjacent Channel Leakage Ratio (ACLR), but cuts DSP consumption by more than half.

- Shared DPD Observation Path: Time-share a single observation receiver (ORx) and coefficient-update engine across all Tx channels rather than adapting concurrently.

- Offload Parameter Estimation: Capture raw input/feedback buffers inside UltraRAM and stream them to the on-chip Quad-Core ARM Cortex-A53 to calculate the matrix inversion (least squares) in software, keeping only the fast forward-filtering pre-distorter in fabric.

- Offload Up/Down Conversion to the Hard Converters: Maximize use of the native Gen-3 RF-ADC/DAC interpolation/decimation filters to avoid implementing multi-stage polyphase FIR filters in programmable logic.

To make this work on a Gen-3 RFSoC (e.g., ZU48DR/ZU49DR), the architecture is partitioned into three distinct domains: Fronthaul & Frequency Domain (Soft PL), Time-Domain DFE & Forward Path (Soft PL), and Adaptation / Calibration (Processing System + Shared Fabric).

The critical architectural trick is time-sharing the observation path and offloading the heavy matrix math to the ARM cores, keeping the programmable logic strictly focused on real-time streaming pipelines.

====================================== FRONT-HAUL & FREQ DOMAIN ======================================

[ 25G/10G Optical ]

│

▼ (GTY Transceivers)

[ eCPRI / O-RAN Deframer (Soft IP) ] ── (C-Plane / Timing Sync) ──► [ IEEE 1588 / PTP Stack in RPU ]

│

▼ (U-Plane IQ: BFP9-14)

[ Decompression / Depruning ]

│

▼ (Frequency-Domain Resource Elements)

[ Soft IFFT Core + Cyclic Prefix Insertion ]  <── (Per-channel 2048/4096-pt, Radix-4 Pipelined)

│

====================================== TIME-DOMAIN DFE PIPELINE ======================================

│ (Time-Domain Baseband @ ~122.88 - 491.52 MSPS)

▼

┌──────────────────────────────────────────┐

│ Soft PC-CFR (Peak Cancellation CFR)      │  <── 2-Stage iterative canceller

└──────────────────────────────────────────┘      (Shared peak detector + pulse generator)

│

▼

┌──────────────────────────────────────────┐

│ Forward DPD Actuator (Soft GMP Engine)   │  <── Low-order Generalized Memory Polynomial

└──────────────────────────────────────────┘      LUT-based magnitude + complex FIR slices

│                                                                                                             ▲

│                                                                                                             │ (Updated Coefficients via AXI-Stream)

▼                                                                                                             │

====================================== HARD CONVERTERS (RF DATA CONVERTERS) =========================

[ Gen-3 RF-DAC Subsystem ]                                                                          │

├── Hard DUC (Mixer + Interpolator 4x/8x)                                               │

└── Direct RF-DAC Core (up to 5-10 GSPS)                                                 │

│                                                                                                              │

▼ (Analog RF Output)                                                                             │

[ RF PA ] ───┬───────────────────────────────────┤

│ (Coupled RF Feedback)                                                              │

▼                                                                                                   │

[ Gen-3 RF-ADC Subsystem (Shared ORx) ]                                                    │

├── Direct RF-ADC Core                                                                            │

└── Hard DDC (Mixer + Decimator)                                                         │

│                                                                                                            │

====================================== SHARED DPD ADAPTATION LOOP ===================================

│ (Digital Down-Converted Feedback)                                                  │

▼                                                                                                           │

[ Capture Buffer Engine (UltraRAM / BRAM) ]                                               │  <── Time-muxed capture across Antennas 0-3

│                                                                                                            │

▼ (AXI DMA / High-Speed Interconnect)                                              │

[ ARM Cortex-A53 APU Subsystem ]                                                            │

├── Buffer Time-Alignment & Fractional Delay                                        │

├── Model Extraction (QR Decomposition / LS)                                        │

└── Stability Check ────────────────────────────────┘

Architectural Subsystem Breakdown

1. Real-Time Forward Path (Programmable Logic)

- eCPRI & Low-PHY: Uses the soft AMD LogiCORE O-RAN Radio Interface. It terminates eCPRI packet framing, unpacks Block Floating Point (BFP) compressed frequency-domain data, and distributes symbols per antenna stream.

- IFFT & CP Insertion: Implemented via Xilinx FFT LogiCORE IPs operating in streaming Radix-4 Burst or Pipelined Streaming I/O mode. For a 4T4R 100 MHz setup, four 2048-pt cores run clocked at ~245.76 MHz or ~491.52 MHz, keeping routing localized.

- Two-Stage PC-CFR: Employs a single peak-detection threshold followed by a circular subtraction buffer containing pre-calculated cancellation pulses. Instead of deep 4- to 6-stage CFR cascades, a 2-stage design reduces PAPR by ~6 dB while slashing BRAM and DSP requirements by over half.

- Pruned GMP DPD Actuator: The forward DPD is implemented as a direct-form filter using a reduced Generalized Memory Polynomial (GMP) model.

  - Memory depth is limited (Q≤2 or 3).

  - Non-linear orders are capped (K≤5, avoiding 7th- and 9th-order expansions).

  - Envelope calculations (|xn|​2) are shared via lookup tables (LUTs) across adjacent memory branches.

2. Converter Stage Offloading (Hard RF-DC Blocks)

- To protect FPGA fabric resources, all heavy multirate filtering is offloaded to the hardened RF-ADC and RF-DAC tiles.

- The soft DPD outputs at baseband or low intermediate sample rate (e.g., 491.52 MSPS). The hardened digital interpolation filters (4x or 8x) and fine NCOs in the RF-DAC shift the signal up into the desired RF band (up to Sub-6 GHz) without burning a single logic LUT or DSP slice.

3. Time-Shared Observation & Software Adaptation (PS + URAM)

- Single Observation Path (ORx): Instead of dedicating 4 feedback ADC receivers to 4 transmit paths, a single high-speed RF-ADC channel is multiplexed via an external RF switch (or internal crossbar). It round-robins across the PAs (e.g., Path 0 for 50 ms, Path 1 for 50 ms).

- Deep Capture Engine: When adapting, a triggered snapshot of simultaneous forward-path input data xn and digitized feedback data yn (typically 4k to 16k samples) is captured into UltraRAM.

- Offloaded Math (ARM APU): The captured frames are pushed over AXI-DMA to the Quad-Core ARM Cortex-A53 running Linux.

  - The ARM cores perform fractional delay alignment, gain normalization, and matrix inversion (e.g., Least Squares / QR Decomposition via standard NEON-accelerated linear algebra libraries).

  - Once new polynomial coefficients are calculated and checked for numerical stability, the APU writes them back to the forward DPD registers via AXI-Lite/AXI-Stream.

  - Why this matters: Eliminates thousands of soft DSP slices that would otherwise be required to build hardware matrix solvers in fabric.

Approximate Logic & Resource Budget (4T4R, 100 MHz BW)

Assuming a XCZU48DR (~4,272 DSPs, ~425k LUTs, ~21 Mb URAM):

| Subsystem | DSP48E2 Slices | Block RAM / URAM | Logic LUTs |
| --- | --- | --- | --- |
| O-RAN / eCPRI / Decomp (4 streams) | 0 | 16 BRAM | ~35,000 |
| Soft IFFT/FFT + CP (4 streams) | ~256 | 32 BRAM | ~25,000 |
| 2-Stage PC-CFR (4 streams) | ~128 | 48 BRAM | ~28,000 |
| Forward DPD Engine (4 streams, pruned GMP) | ~1,200 – 1,600 | 16 BRAM + 8 URAM | ~70,000 |
| Capture Engine & DMA Interconnect | 0 | 16 URAM | ~15,000 |
| Total Estimated Utilization | ~35% – 45% | ~25% – 35% | ~40% – 50% |

Margin: This keeps total device utilization under 60%, which is essential on UltraScale+ architectures to reliably hit timing closure at 300+ MHz clock frequencies.

Break down the clock domain crossing and distribution strategy for this Gen-3 RFSoC DFE design.

In an RFSoC-based 4T4R 100 MHz radio unit, managing clock distribution and Clock Domain Crossings (CDCs) is critical. Running wide datapath algorithms like DPD and CFR alongside high-speed SerDes and RF data converters requires balancing timing closure with minimal latency jitter.

| Domain Name | Frequency | Source / Root | Primary Consumers |
| --- | --- | --- | --- |
| clk_eth_mac | 156.25 MHz / 390.625 MHz | GTY Reference (via optical SFP28) | 25G Ethernet MAC, eCPRI packet parser, IEEE 1588 time stamping. |
| clk_sample (Baseband) | 122.88 MHz (or 245.76 MHz) | On-board low-jitter jitter cleaner (e.g., LMK04828 / LMX2594 via MMCM) | IFFT/FFT cores, Cyclic Prefix insertion/removal, Frequency-domain beamforming. |
| clk_dfe (Oversampled) | 491.52 MHz (1x sample/clock) or 245.76 MHz (2x samples/clock) | Synthesized from clk_sample via dedicated PLL/MMCM | CFR peak detection, DPD actuator (GMP filter), DPD capture buffer. |
| clk_axi_sys | 100 MHz / 150 MHz | PS FCLK (Processing System) | AXI4-Lite control registers, DPD coefficient updates, health monitors. |
| clk_aximm_dma | 250 MHz / 300 MHz | Dedicated MMCM | AXI4-MM DMA engines streaming captured DPD buffers from URAM to ARM DDR. |
| clk_rf_fabric | 245.76 MHz / 491.52 MHz | RF-ADC / RF-DAC tile PLLs | Converter AXI4-Stream interface logic, converter crossbars. |

- Jitter & MTS Synchronization: Use the RFSoC Multi-Tile Sync (MTS) IP. The external clock generator feeds synchronized reference clocks and periodic SYSREF pulses to all RF-ADC/DAC tiles to eliminate inter-tile skew.

- Synchronous MMCM Synthesis: Derive clk_sample and clk_dfe from the exact same PL-side MMCM using integer divide ratios (1×,2×,4×). Because they share an MMCM root, their phase relationship is deterministic, eliminating non-deterministic CDC wander.

Clock Domain Crossing (CDC) Implementation Strategy

1. Fronthaul to Baseband: clk_eth_mac → clk_sample

- Nature: Asynchronous. Ethernet recovers its clock from the optical line (or local oscillator depending on the SerDes topology), while baseband runs on the radio sample reference.

- Mechanism: Asynchronous AXI4-Stream FIFO (axis_data_fifo).

- Implementation:

  - Ingest eCPRI U-Plane messages across the asynchronous boundary.

  - Use a programmable water-mark threshold inside the FIFO to absorb packet arrival jitter without underflowing the IFFT input stream.

2. Baseband to DFE Forward Path: clk_sample → clk_dfe

- Nature: Mesochronous / Integer-Harmonic Synchronous. Both clocks originate from the same MMCM (122.88 MHz and 491.52 MHz or 245.76 MHz).

- Mechanism: Rate Transition via Multi-Cycle Path (MCP) or Lightweight Synchronous FIFO.

  - If operating at 491.52 MHz with 1 sample per clock, use a simple fractional-rate enabler (valid high every 4th clock) or a tiny 4-deep synchronous FIFO.

  - Define timing constraints (set_multicycle_path) cleanly in the XDC file to prevent Vivado from trying to close edge-to-edge hold time on unrelated edges:

set_multicycle_path -setup -from [get_clocks clk_sample] -to [get_clocks clk_dfe] 4

set_multicycle_path -hold  -from [get_clocks clk_sample] -to [get_clocks clk_dfe] 3

DFE Fabric to Hard Data Converters: clk_dfe → clk_rf_fabric

- Nature: Synchronous (with careful phase matching). The RF-DAC tile generates its internal sampling clock and exposes a divided user-clock (clk_rf_fabric) to the PL.

- Mechanism: Match clk_dfe to clk_rf_fabric directly.

  - To avoid high-frequency CDCs at the DAC boundary, configure the RF-DAC IP core's fabric clock output to drive the DFE datapath directly via a BUFG, or lock the PL MMCM to the same reference.

  - If frequencies match and clocks are phase-aligned via MTS, the interface operates as a direct register-to-register synchronous transfer.

4. DFE Capture to ARM PS: clk_dfe → clk_aximm_dma → clk_axi_sys

- Nature: Asynchronous. Fast streaming radio domain to system/processor interconnect.

- Mechanism: True Dual-Port UltraRAM (TDP URAM).

  - Port A (Write): Driven by clk_dfe. Writes captured forward path xn and feedback yn samples on a continuous circular ring buffer until triggered.

  - Port B (Read): Driven by clk_aximm_dma (250 MHz). Read directly by an AXI Central DMA (CDMA) or scatter-gather DMA engine.

  - Trigger/Pointer Synchronization: Use a 2-stage synchronizer (xpm_cdc_single or xpm_cdc_gray) for the capture trigger and write-done status flags.

5. PS Coefficient Updates: clk_axi_sys → clk_dfe

- Nature: Quasi-Static / Asynchronous. The ARM APU writes DPD polynomial weights infrequently (e.g., once every 50–100 ms).

- Mechanism: Shadow Register Handshake (xpm_cdc_handshake).

  - The processor writes the full set of complex weights into an AXI-Lite accessible shadow register set on clk_axi_sys.

  - Once the entire coefficient set passes a software stability check, a single "commit" toggle is synced across to clk_dfe.

  - The DFE logic latches all new coefficients simultaneously into the active GMP pipelines during a frame boundary or symbol gap to avoid phase discontinuities.

High-Frequency (491.52 MHz) Timing Closure Tactics

Closing timing at 491.52 MHz across thousands of DSP and BRAM primitives in a soft Gen-3 implementation requires strict floorplanning and structural constraints:

- 2x Over-Clocking vs 2-Sample Vectoring:

  - Running 1 sample/clock at 491.52 MHz stresses routing channels and logic levels between DSPs.

  - Alternative: Run the soft DFE datapath at 245.76 MHz with a 2-sample-wide vector (SIMD/Parallel). This cuts the clock requirement in half, gives an 8.13 ns period instead of 2.03 ns, and simplifies Vivado placement while keeping the net throughput identical (491.52 MSPS aggregate).

- Dedicated Pblocks: Constrain each antenna’s DPD engine to the specific Clock Region and DSP column directly adjacent to its corresponding RF-DAC/ADC tile to prevent long routing tracks across SLR or inter-clock boundaries.

- Maximized Internal DSP Pipelining: Ensure AREG=1/2, BREG=1/2, MREG=1, and PREG=1 are fully enabled in all DSP48E2 instantiations. Never infer combinatorial paths between DSP slices across the 491.52 MHz domain.

Here is the comparative trade-off breakdown between running a Single-Sample Serial Datapath at 491.52 MHz versus a 2-Sample Parallel (SIMD) Datapath at 245.76 MHz on a Gen-3 UltraScale+ RFSoC.

| Metric / Dimension | Single-Sample (491.52 MHz) | 2-Sample Parallel (245.76 MHz) | Practical Verdict |
| --- | --- | --- | --- |
| Clock Period (Tclk) | 2.034 ns | 4.069 ns | 245.76 MHz provides 2x the setup-time budget, radically easing routing delays. |
| Vivado Timing Closure Effort | Very High / Fragile | Low to Moderate | 491 MHz requires deep hand-pipelining and tight Pblocks; small RTL changes can break closure. |
| DSP48E2 Primitive Count | Baseline (1×) | 1.8×-2.0× | Parallel processing duplicates the complex multiplier pipelines across two simultaneous samples. |
| LUT / Flip-Flop Overhead | Baseline (1×) | ≈1.5×-1.8× | Vector routing, cross-sample multiplexing, and wider control busses increase logic footprint. |
| Pipeline Latency (Time) | Lowest (shorter absolute time) | Slightly Higher (+20–40%) | Even though pipeline stages are fewer per sample, wider alignment registers add fractional nanoseconds. |
| Memory / Delay Line Complexity | Simple shift registers (SRL16E/SRL32E) | Stride-2 interleaved memory buffers | 491 MHz maps directly to compact SRL primitives; parallel paths require multi-port RAM or ping-pong buffers. |
| CFR Peak Cancellation Complexity | Direct serial peak detection | Inter-sample peak detection logic | Identifying peaks spanning adjacent samples in the same clock edge adds combinatorial logic. |

Deep Dive: How the Blocks Are Impacted

1. DPD Actuator (GMP Filter)

- At 491.52 MHz (Serial):

  - One complex multiplication per polynomial branch per clock.

  - You can leverage the internal DSP cascade paths (ACOUT → ACIN, PCOUT → PCIN) without spilling routing into general fabric interconnect.

  - The Trap: The envelope lookup tables (e.g., calculating |xn|​2 to address the polynomial gain LUT) must complete address calculation, memory read, and feed the DSP multiplier within ~2 ns.

- At 245.76 MHz (2-Sample Parallel):

  - Sample x2n and x2n+1 arrive simultaneously.

  - Every memory polynomial term xn-m⋅|xn-m-l|​k requires two physical complex multiplier chains running side-by-side.

  - The DSP budget for a 4T4R pruned GMP engine jumps from ~1,200 DSPs to ~2,200–2,400 DSPs. On a ZU48DR (~4,272 DSPs), this consumes over 55% of the entire die for DPD alone.

2. Crest Factor Reduction (PC-CFR)

- At 491.52 MHz:

  - Simple serial comparator scans peak magnitude against the PAPR threshold. When a peak is found, a pre-stored cancellation pulse is subtracted via a single circular buffer.

- At 245.76 MHz:

  - Both x2n and x2n+1 must be evaluated concurrently.

  - If both exceed the threshold, or if a peak falls between sample boundaries, the logic must determine the true local maximum across adjacent parallel samples. Applying overlapping cancellation pulses requires multi-phase cancellation adders, significantly bloating logic LUT usage.

3. Data Converter Interface (RF-DAC / RF-ADC)

- The Gen-3 RF-DAC native AXI4-Stream interface naturally prefers a multi-word interface depending on the configured interpolation mode:

  - For instance, an RF-DAC interpolating 8x to 3.932 GSPS accepts 491.52 MSPS.

  - The RF-DAC IP can expose either 1 sample/clock at 491.52 MHz or 2 samples/clock at 245.76 MHz.

  - Using 245.76 MHz natively matches the native FIFO interface of the RF-DC core with virtually zero clock-rate transition logic at the boundary.

The Recommended Hybrid Strategy

Rather than choosing an all-or-nothing approach across the entire chip, the most resource-efficient industry architecture for Gen-3 RFSoC uses a split-frequency pipeline:

[ Baseband / IFFT ] ──► [ 2-Stage PC-CFR ] ─────► [ DPD Actuator ] ──────────► [ RF-DAC Tile ]

@ 122.88 MHz                         @ 245.76 MHz                      @ 491.52 MHz Direct Hardened

(Low DSP usage) (2-Sample Parallel) (Single-Sample Serial) Interpolation Easier peak-search routing Saves ~1,000 DSP slices

  Keep CFR at 245.76 MHz (2-Sample): Peak search algorithms and circular subtraction pulse FIFOs struggle with 2.0 ns setup times when routing across wide BRAM/LUT structures. 245.76 MHz provides comfortable margin.

  Serialize the DPD Actuator to 491.52 MHz (1-Sample): Convert from 2-sample to 1-sample right before the DPD engine. Because DPD is strictly feed-forward FIR-like structures, you can take full advantage of DSP48E2 dedicated systolic cascade chains (PCOUT → PCIN), which run natively up to 700+ MHz on UltraScale+ without using general routing fabric. This protects your critical DSP slice budget.