# Step: FPGA high level architecture

Relevant for new projects on new boards. FPGA family/vendor and specific part selection. Define the overall FPGA architecture, select the device family, and estimate early resource and power requirements.

Power estimation using Early Power Estimation Tools. First utilization estimation using similarity to previous designs and some estimation for the required logic and other FPGA resources based on the designer knowledge and experience.

For DSP logic, some early MATLAB work is needed. Software aspects need to be also taken in consideration.

Tools and methods:

Excel-based power estimation sheets

Vendor Early Power Estimation (EPE) tools

MATLAB for DSP modeling

Engineering judgment based on prior designs

AI Opportunity

- Power estimation agent – Automates EPE tool usage, extracts results, and generates power-budget reports.

- Utilization estimation agent – to evaluate the required logic resources based on the functional unit parameters. Predicts LUT/FF/BRAM/DSP usage from functional parameters.

- FPGA selection Agent – Compares device families, cost, power, transceivers, memory interfaces, and recommends optimal parts.

# Step: FPGA pinout design

Pinout mapping to fit the FPGA requirements and the board limitations. Map logical signals to physical pins while respecting bank voltages, I/O standards, and board constraints.

AI Opportunity

- FPGA Pad design agent – generate pad mapping based on the requirements. Creates pinout proposals based on interface requirements and board schematics.

- FPGA Pad agent – analyze the pin mapping against the FPGA limitations and power domains and banks, clocks and special pins. Checks bank voltages, special pins, clock‑capable pins, and flags violations.

# Step: FPGA partition and clock domains design

Design the FPGA architecture partition. Define module boundaries, clock domains, and data‑flow architecture. What design units will be used to implement the FPGA functionality. Design of the clock domains according to the FPGA interfaces and functional requirements.

AI Opportunity

- Partition agent – analyze the most efficient and optimize clock domain and design partition by reducing the clock domain crossing. Suggests module grouping to reduce CDC complexity and improve timing.

- Clock‑Domain Planner agent - Recommends clocking structure, PLL usage, and CDC synchronization strategies.

# Step: Micro architecture design

Translate system-level requirements into detailed module-level architecture.

Tools

Usually some diagram sketch like MS-Visio and Drow.io

Documented in word doc format

AI Opportunity

- Architect agent – analyze the proposed architecture and detects potential bottlenecks and not optimal structures and data flow for the considered clock rates and data throughput

# Step: Fast Protocols and transceivers

The typical FPGA has several fast transceivers interfaces that implement gigabit protocols.

Some of them: DDR3/4/5. PCIe, CPRI, Aurora, 10/25G ETH, JESD204B

Tools:

Typicaly, the transceivers are embedded as part of the specific IP-Core for the used protocol. The parameters for the transceiver and the protocol are implemented in the vendor tools.

AI Opportunity

- DDR control agent

- Aurora Control agent

- GETH control agent

- PCIe control agent

- JESD control Agent

# Step: SOC and Soft core design

In SoC FPGAs the design involves the design of the SoC system usually with vendor specific tools. The system design provides the ability to design the AXI interfaces around the CPU and the various data paths to external memories and the FPGA logic.

In Vivado, Block Design EDA tool is used with graphical user interface to build the system.  The design is fully described by TCL file

Tools:

Vivado block design

Vitis

Questa System design

AI Opportunity

- TCL design agent

- Vivado BD design agent

- Vivado BD analysis agent

- Quartus qsys design agent

- Quartus qsys analysis agent

# Step: RF design

In FPGAs that include RF ADC/DAC, this step is used to design and configure the RF units – ADC and DAC. The configuration is loaded to the design by configuring the ADC/DAC parameters in the RF Converters unit.

Tools:

Vivado

Vitis

Questa System design

AI Opportunity

- RF Configuration agent – based on the requirements performs the RF unit configuration

- RF Configuration analysis agent – reads the RF configuration and identifies gaps in the configuration against the requirements.

- Xilinx Frequency planner agent – use the Xilinx frequency planner tool and verify the RF converter design against it.

# Step: Logic design

RTL code design – writing the RTL code that implements the microarchitecture and the required design.

Using HLS methods to generate logic.

Tools:

Code editors with AI support – VSCode with Cline, Continue etc..

Vitis HLS compiler

AI Opportunity

- RTL Agent – providing VHDL/Verilog code based on the requirements

- HLS Agent – providing VHDL/Verilog code based on the requirements

- Working with AI add-ons to expiate the process

- Design Review Agent – review the code to verify it follows the rules and good engineering practices

# Step: IP-Core design

Generating the vendor Cores that are integrated in the design. Units like FIFO, memories, FFT , filters and other Cores from vendor.

Design the interface for 3’rd party IP-Cores embedded in the design

Tools:

CoreGen and vendor Core library

AI Opportunity

- IP-Core agent – operate the vendor tools to generate the IP Cores

# Static Analysis, Lint & Coding-Standard Compliance

Static code analysis

Tools:

Mentor tools, vLint

AI opportunities:

- Lint-waiver agent - propose justifications and route waivers for review.

- Coding-standard agent - auto-fix style violations and flag anti-patterns (e.g., latches, async resets, mixed blocking/non-blocking).

- Rule-deck authoring agent - turn natural-language design rules into Spyglass goals.

# Step: Logic Simulation and Verification

The design is simulated with logic simulation to validate it full correctness. The simulation runs using the simulation software like Questa and structured as tests to validate the different design sanction.

Some IP cores are used as BFM – Functional Bus model that emulates the subset of the functionality of complex IP-cores. CPU operations are also usually done as emulation to the actual CPU, either external CPU or embedded CPU.

Advanced simulation may use system Verilog and UVM practices.

Simulation is usually follow two main simulation paths. Control logic that validates the correct operation of the logic with different operation and control conditions.

Digital processing logic that validates the correct mathematical operation of the units and use vector driven simulation. Input vector is driving the design and the result Is verified against the golden vector. All the simulation vectors are provided by the Matlab/Python simulation.

Tools:

Logic simulator – ModelSim/Questa

CDC tools: vSync, Mentor CDC

AI Opportunity

- CDC Agent – operates the CDC design and prepares the scripts for using in the CDC tools

- Working with AI add-ons to expiate the process

- Sim Plan Agent – plan of the test bench and the testing methods to validate the design.

- BFM Agent – generates code

- Python simulation agent

- TCL simulation agent

- SV agent -

- UVM Plan agent -

- UVM agent -

- Sim Result analysis agent -

# Formal Verification & Property Checking (optional)

Formal verification control logic, FSMs, arbiters, and CDC paths

Tools:

Questa Formal, Synopsys VC Formal

AI opportunities:

- SVA-property generator agent — derive assertions from the spec / register file / FSM diagrams.

- Cover-point agent — propose functional coverage points from the microarchitecture document.

- Formal-result analysis agent — triage counter-examples and suggest constraint fixes.

# Step: Design of software and embedded Software

Design the software interfaces, register file and the IDD. Identify the embedded software in the FPGA and design the bare-metal software (separate design flow).

Tools:

Usually documented in plain MS-Word document and MS-Excel tables for IDD. Software instructions for API are in MS-Word and Excel

AI Opportunity

- Memory map design agent – design the memory map of the FPGA

- IDD agent – design the registers for the design according to the architecture and prepare the IDD document.

- Register agent – design the code memory map and bit-field structure for the registers in the design

# Step: Version Control CI/CD, Build Automation & Regression

During the design process GIT is used to version control the FPGA code.

During the whole FPGA design cycle, the FPGA build and regression testing to provide the FPGA for integration and release continue.

Tools:

GIT, SmartGit, GitLab, Jenkins, Make/CMake, tclArgParser, Hancho, custom Python build frameworks, container images of Vivado/Quartus for reproducibility.

AI Opportunity

- Git agent – used to do commits with the commit message

- Build-script agent — generate Vivado/Quartus Tcl build scripts from a project manifest.

- Regression-dashboard agent — summarize nightly sim+impl results, flag regressions, attribute to recent commits.

- Reproducibility agent — verify a release can be rebuilt bit-identically.

# Step: Design for Test

Adding test and debug logic that will be used during the FPGA POD, debug and advanced testing as well as after deployment. Some logic sould be added to support the ATP testing.

Logic for logging in different stages should be added to record operational logs also during product deployment.

During this stage the FPGA remote update should be also considered and design should be supporting when this is part of the requirements.

Tools:

Usually done as design documents that used as part of the microarchitecture and FPGA coding.

Tools:

AI Opportunity

- DFT Analysis agent -

- DFT Agent – propose and plan the specified structures in the FPGA that will be used for testing and debugging

# Step: FPGA Design documents and Reviews

Essential and required documents preparation. FDD – firmware design document, and other relevant artifacts. The document includes the microarchitecture design, design requirements and comply table, register file description, state machines, timing diagrams and verbal description of the various units and operation principles and logic.

ICD (Interface Control Document) is also required document as part of this design step

Preparation of the mandatory design reviews:

SRR – System Requirement review

PDR/CDR – Design reviews, PDR in early stages, CDR in advanced stages.

Tools:

MS-Word and Excel, MS-Visio/Draw.io, power point

AI Opportunity

- SRR Agent - prepares the basic structure and some content for the SRR review

- PDR Agent - prepares the basic structure and some content for the PDR review

- CDR Agent – prepares the basic structure and some content for the CDR review

- FDD Agent – prepares the basic structure and content for the FDD document

- ICD Agent – prepares the ICD document

# Step: FPGA Fit (Synthesis + P&R)

This step is the main step when the design meets the FPGA. During this step, the whole design is synthesized to the FPGA target library and then placed and mapped on the FPGA logic gate structures and resources. Then the FPGA is routed to get the final fully routed FPGA image.

Power estimation using Early Power Estimation Tools with the back annotation file generated from the Quartus or Vivado

Timing back annotation file may be generated that may be used for simulation with real timing data.

The process is timing driven, the timing constraints file defines the timing requirements on the different clocks and interfaces in the design.

The process is also placement constraint driven (in case placement constraints are used, which is not recommended for typical design)

Tools:

Vivado

Vitis

Quartus

AI Opportunity

- Timing Constraint files agent – prepares the timing constraint file to be used during the P&R process and later during the timing closure step.

- Placement constraint agent

- Vivado script agent – prepare scripts to implement the design

- Quartus script agent – prepare scripts to implement the design

- Synth-PAR report analysis agent – very important agent that analyze the

# Step: FPGA Bit Generation

Final step in FPA generation process is to generate the binary bit file that may be loaded to the FPGA and the flash image in case the FPGA is programmed via flash. In case the FPGA is loaded by the CPU, the configuration file is embedded in the file system.

AI Opportunity

- Bit gen agent – generate script or directly operate the design IDE to generate the bit file.

# Step: Timing Closure

Timing closure is the process of testing the design timing against the timing constraints for all corners (PVT – process, voltage, temperature) and all the different intefaces.

The preparation of the constraints is crusual, as it will define the operation of the design. Any path that missing constraint coverage (incomplete coverage) may lead to design

AI Opportunity

- Transceiver constraint file agent

- DDR constraint file agent

- Pad constraint file agent

- Timing results analysis agent

- Timing test commands agent

- Timing fix proposal agent

# Step: Testing POD and software testing

Testing the FPGA full capabilities and requirements and full operational envelop.

Preparing the POD document and the ATR documentation

Preparation of version released document and release the design to the project.

AI Opportunity

- POD Agent – preparing the POD test list that following the SRR and the system requirement document.

- POD Analysis agent – analysis of the logs from the POD tests to identify possible problems and ways to address the issues

- FVD Document agent – building the FVD (Firmware Version Document) document ready to release including documentation of the version building and restore

# Step: ATP Testing

The ATP testing stage is during the production stages. Manual or Automatic test of the system including the FPGA. The tests are targeted to test the specific unit performance and proper operation, not the design functional requirements that were tested in the POD stage. Usually done by production team not the design team. The design team prepares the documentation for the ATP.

AI Opportunity

- ATP documentation agent – to build efficient ATP document

- ATR analysis agent – receive the results and compose the results ATR document

- ATP debug agent – based on the ATR results to propose possible fix options. Based also on previous testing.

# Step: Advanced Integration support

Integration in full system and real-world conditions and corner cases

Tools:

Working with test application software that emulates the application software, or with the actual application layer software that operates and interacts with the FPGA.

The user run specific scripts, usually python or Matlab to analyze recording files and log files and other artifacts to analyze the results to validate and verify the correct operation of the system and detect and debug ab-normal results and system states.

To debug, in-system tools are used like signalTap or Chip-scope. These units are inserted to the design and the results are read from the test unit by using the vendor dedicated tools which are part of Quartus and Vivado

AI Opportunity

- ChipScope agent

- SignalTap Agent

- ChipScope analysis agent

- SignalTap analysis Agent

- Debug script agent

- Result analysis agent

# Additional design steps

Additional stages may apply buy are very specific for certain designs

1. Algorithm & Bit-True Modeling (before RTL) – algorithmic flow not considered as part of the FPGA flow

This is the gap I'd close first, given the DSP nature of most of your work. Today the document mentions MATLAB briefly inside "High level architecture" and again as the source of golden vectors in simulation, but the algorithm-development / fixed-point conversion stage is a discipline of its own and usually consumes 10–20% of a DSP-heavy project.

Purpose: Floating-point algorithm prototyping → fixed-point conversion → quantization-error analysis → generation of golden reference vectors and a bit-true model that the RTL must match.

Tools: MATLAB (Fixed-Point Designer, HDL Coder), Python (NumPy, SciPy, MyHDL/Amaranth for bit-true), Simulink, Octave.

AI opportunities:

- Fixed-point conversion agent — propose word/fraction widths from dynamic-range analysis and target SQNR.

- Quantization-noise analysis agent — compare floating vs. fixed-point error budgets across the signal chain.

- Golden-vector generator agent — produce test stimuli + expected outputs aligned with the verification plan.

- Algorithm-to-microarchitecture agent — suggest pipelining, parallelism, and resource trade-offs from the algorithm structure.

4. Logic Equivalence Checking (LEC)

Belongs between Synthesis/P&R and Timing Closure. Mandatory in DO-254 / safety-critical flows; nice-to-have everywhere else.

Tools: Conformal LEC, Formality, OneSpin 360 EC.

AI opportunities:

- LEC setup agent — generate mapping files and key-point setups.

- Mismatch analysis agent — classify mismatches (real bug vs. synthesis transform vs. constant propagation) and propose fixes.

5. Safety & Certification Compliance

If any of your projects touch DO-254 (aerospace), ISO 26262 (automotive), or IEC 61508 (industrial), the whole flow gains a parallel "evidence" track that the current document doesn't reflect.

Activities: DAL/SIL allocation, traceability matrix, FMEA/FMEDA, independent verification, elemental analysis, tool-qualification artifacts.

AI opportunities:

- Traceability agent — maintain requirements ↔ design ↔ tests ↔ coverage links.

- FMEDA agent — propose failure modes per module and compute diagnostic coverage.

- Tool-qualification agent — assemble TQL/TCL evidence packages.

- Elemental-analysis agent — produce post-implementation gate-level analysis reports.

6. Security & Anti-Tamper Engineering

Particularly relevant for defense work — and increasingly for commercial too.

Activities: Bitstream encryption & authentication, eFUSE / RoT key provisioning, secure boot, anti-readback, side-channel-resistant primitives, asset/threat modeling.

AI opportunities:

- Threat-model agent — derive an FPGA-specific threat model from the system architecture.

- Bitstream-protection agent — configure encryption/authentication options per Vivado/Quartus security flow.

- Side-channel review agent — flag DPA-sensitive logic (crypto cores, key handling).

7. Partial Reconfiguration / Dynamic Function eXchange

A first-class flow on UltraScale+, Versal, and Agilex when you need to swap accelerators at runtime, time-share resources, or remote-update a region without halting the rest.

Tools: Vivado DFX flow, Quartus Partial Reconfiguration flow.

AI opportunities:

- DFX partition agent — propose reconfigurable regions and pblock floorplans.

- Decoupling-logic agent — generate isolation logic and handshake protocols between static and reconfigurable regions.

- RM (Reconfigurable Module) build agent — automate the per-RM implementation runs.

9. Bring-up / First-Light

Currently absorbed into "Advanced Integration support," but the first power-on of a new board with a new design is a distinct phase: clocking, configuration mode, JTAG, basic-of-life tests, then progressive enablement.

AI opportunities:

- Bring-up plan agent — generate a staged checklist (power → config → clocks → resets → interfaces → datapath).

- First-light debug agent — correlate symptoms (no DONE, MMCM unlocked, link not up) with likely root causes.

11. Vector-Based Power Analysis

You have Early Power Estimation at the front and back-annotated EPE after P&R. The missing piece is dynamic, activity-driven power using simulation VCD/SAIF, which gives much more realistic numbers than the default switching-activity assumptions.

AI opportunities:

- SAIF generation agent — instrument representative simulation runs and produce activity files.

- Power hotspot agent — read XPA reports and propose clock-gating / data-gating opportunities.

12. Thermal, Mechanical & Reliability Sign-off

Often owned by hardware, but the FPGA team should provide inputs (junction-temperature targets, derating, package thermal model) and validate against measured data.

AI opportunities:

- Thermal-budget agent — propagate ambient + airflow + package θJA into Tj predictions and compare to spec.

- Derating agent — apply mil/aerospace derating curves to vendor max-frequency/voltage data.

# Design cycle

The following table summarizes the design cycles, each step with the effort percentage of the whole design cycle and the complexity/risk of each step.

| Step | Of total cycle [%] | Complexity/Risk [5 – high, 0-low] |
| --- | --- | --- |
| 1. FPGA high level architecture | 4 | 4 |
| 2. FPGA pinout design | 1 | 3 |
| 3. FPGA partition and clock domains | 2 | 4 |
| 4. Micro architecture design | 6 | 4 |
| 5. Fast Protocols and transceivers | 4 | 5 |
| 6. SOC and Soft core design | 4 | 3 |
| 7. RF design | 2 | 5 |
| 8. Logic design | 17 | 3 |
| 9. IP-Core design | 2 | 2 |
| 10. Static Analysis, Lint & Coding standards | 1 | 1 |
| 11. Logic Simulation and Verification | 22 | 5 |
| 12. Formal Verification & Property Checking | 2 | 3 |
| 13. Design of software and embedded SW | 3 | 3 |
| 14. Version Control, CI/CD, Build & Regression | 2 | 2 |
| 15. Design for Test | 2 | 3 |
| 16. FPGA Design documents and Reviews | 5 | 2 |
| 17. FPGA Fit (Synthesis + P&R) | 3 | 4 |
| 18. FPGA Bit Generation | 1 | 1 |
| 19. Timing Closure | 6 | 5 |
| 20. Testing POD and software testing | 5 | 4 |
| 21. ATP Testing | 1 | 2 |
| 22. Advanced Integration support | 5 | 5 |

# AI Agent list

The following table summarizes the AI agents that will be used and the relevant stage in the design process

| # | Agent | Stage | Notes |
| --- | --- | --- | --- |
| 1 | Power estimation agent | 1. FPGA high level architecture | Drives EPE tools, extracts results, generates power-budget reports. |
| 2 | Utilization estimation agent | 1. FPGA high level architecture | Predicts LUT/FF/BRAM/DSP usage from functional parameters. |
| 3 | FPGA selection agent | 1. FPGA high level architecture | Compares device families on cost, power, transceivers, memory I/F; recommends optimal part. |
| 4 | FPGA pad design agent | 2. FPGA pinout design | Generates pinout proposals from interface requirements and board schematics. |
| 5 | FPGA pad analysis agent | 2. FPGA pinout design | Checks bank voltages, special pins, clock-capable pins; flags violations. |
| 6 | Partition agent | 3. FPGA partition and clock domains | Suggests module grouping to reduce CDC complexity and improve timing. |
| 7 | Clock-domain planner agent | 3. FPGA partition and clock domains | Recommends clocking structure, PLL usage, and CDC synchronization strategies. |
| 8 | Architect agent | 4. Micro architecture design | Detects bottlenecks and non-optimal structures vs. target clock rates and throughput. |
| 9 | DDR control agent | 5. Fast Protocols and transceivers | Drives DDR3/4/5 controller IP configuration and constraints. |
| 10 | Aurora control agent | 5. Fast Protocols and transceivers | Generates Aurora link configuration and lane setup. |
| 11 | GETH control agent | 5. Fast Protocols and transceivers | Configures 10/25G Ethernet MAC/PHY IP. |
| 12 | PCIe control agent | 5. Fast Protocols and transceivers | Configures PCIe root/endpoint IP and link parameters. |
| 13 | JESD control agent | 5. Fast Protocols and transceivers | Configures JESD204B/C for ADC/DAC links. |
| 14 | TCL design agent | 6. SOC and Soft core design | Generates Tcl block-design scripts. |
| 15 | Vivado BD design agent | 6. SOC and Soft core design | Builds Vivado Block Design from system spec. |
| 16 | Vivado BD analysis agent | 6. SOC and Soft core design | Reviews existing BD for gaps and inefficiencies. |
| 17 | Quartus QSys design agent | 6. SOC and Soft core design | Builds Quartus Platform Designer (QSys) systems. |
| 18 | Quartus QSys analysis agent | 6. SOC and Soft core design | Audits QSys interconnect and component setup. |
| 19 | RF configuration agent | 7. RF design | Configures RF-ADC/DAC parameters from requirements. |
| 20 | RF configuration analysis agent | 7. RF design | Reads RF converter setup and flags gaps vs. requirements. |
| 21 | Xilinx frequency planner agent | 7. RF design | Uses the vendor frequency planner and verifies the converter design. |
| 22 | RTL agent | 8. Logic design | Produces VHDL/Verilog from microarchitecture and requirements. |
| 23 | HLS agent | 8. Logic design | Generates HDL via HLS from C/C++ models. |
| 24 | Design review agent | 8. Logic design | Reviews code against coding rules and engineering best practices. |
| 25 | IP-core agent | 9. IP-Core design | Drives vendor IP-generation tools and integrates results. |
| 26 | Lint-waiver agent | 10. Static Analysis, Lint & Coding standards | Proposes justifications and routes waivers for review. |
| 27 | Coding-standard agent | 10. Static Analysis, Lint & Coding standards | Auto-fixes style violations and flags anti-patterns (latches, async resets, mixed blocking). |
| 28 | Rule-deck authoring agent | 10. Static Analysis, Lint & Coding standards | Turns natural-language rules into Spyglass goals / lint decks. |
| 29 | CDC agent | 11. Logic Simulation and Verification | Drives CDC tools and prepares scripts for analysis. |
| 30 | Sim-plan agent | 11. Logic Simulation and Verification | Plans testbench architecture and test methodology. |
| 31 | BFM agent | 11. Logic Simulation and Verification | Generates Bus Functional Models for complex IP. |
| 32 | Python simulation agent | 11. Logic Simulation and Verification | Builds Python-based reference / cosim flows. |
| 33 | Tcl simulation agent | 11. Logic Simulation and Verification | Generates and runs simulator Tcl scripts. |
| 34 | SV agent | 11. Logic Simulation and Verification | Produces SystemVerilog testbench code. |
| 35 | UVM plan agent | 11. Logic Simulation and Verification | Drafts UVM verification plan: agents, sequences, scoreboards. |
| 36 | UVM agent | 11. Logic Simulation and Verification | Generates UVM components from the plan. |
| 37 | Sim result analysis agent | 11. Logic Simulation and Verification | Analyzes simulation logs and waves; classifies failures. |
| 38 | SVA-property generator agent | 12. Formal Verification & Property Checking | Derives assertions from spec, register file, and FSM diagrams. |
| 39 | Cover-point agent | 12. Formal Verification & Property Checking | Proposes functional coverage points from the microarchitecture. |
| 40 | Formal-result analysis agent | 12. Formal Verification & Property Checking | Triages counter-examples and suggests constraint fixes. |
| 41 | Memory map design agent | 13. Design of software and embedded SW | Designs FPGA memory map. |
| 42 | IDD agent | 13. Design of software and embedded SW | Designs register set per architecture and produces the IDD document. |
| 43 | Register agent | 13. Design of software and embedded SW | Designs register memory map and bit-field structure. |
| 44 | Git agent | 14. Version Control, CI/CD, Build & Regression | Performs commits with meaningful messages. |
| 45 | Build-script agent | 14. Version Control, CI/CD, Build & Regression | Generates Vivado/Quartus Tcl build scripts from a project manifest. |
| 46 | Regression-dashboard agent | 14. Version Control, CI/CD, Build & Regression | Summarizes nightly sim+impl results; flags regressions and attributes them to recent commits. |
| 47 | Reproducibility agent | 14. Version Control, CI/CD, Build & Regression | Verifies a release can be rebuilt bit-identically. |
| 48 | DFT analysis agent | 15. Design for Test | Reviews existing DFT coverage and proposes improvements. |
| 49 | DFT agent | 15. Design for Test | Proposes and plans test/debug structures in the FPGA. |
| 50 | SRR agent | 16. FPGA Design documents and Reviews | Prepares structure and content for the SRR review. |
| 51 | PDR agent | 16. FPGA Design documents and Reviews | Prepares structure and content for the PDR review. |
| 52 | CDR agent | 16. FPGA Design documents and Reviews | Prepares structure and content for the CDR review. |
| 53 | FDD agent | 16. FPGA Design documents and Reviews | Prepares structure and content for the Firmware Design Document. |
| 54 | ICD agent | 16. FPGA Design documents and Reviews | Prepares the Interface Control Document. |
| 55 | Timing-constraint file agent | 17. FPGA Fit (Synthesis + P&R) | Prepares timing constraints for P&R and timing closure. |
| 56 | Placement-constraint agent | 17. FPGA Fit (Synthesis + P&R) | Proposes pblocks / location constraints where needed. |
| 57 | Vivado script agent | 17. FPGA Fit (Synthesis + P&R) | Builds Vivado implementation scripts. |
| 58 | Quartus script agent | 17. FPGA Fit (Synthesis + P&R) | Builds Quartus implementation scripts. |
| 59 | Synth/P&R report analysis agent | 17. FPGA Fit (Synthesis + P&R) | Analyzes synthesis and P&R reports for utilization, congestion, timing issues. |
| 60 | Bit-gen agent | 18. FPGA Bit Generation | Generates the bitstream / flash image via vendor IDE or scripts. |
| 61 | Transceiver constraint file agent | 19. Timing Closure | Generates timing constraints for transceiver paths. |
| 62 | DDR constraint file agent | 19. Timing Closure | Generates DDR-specific timing constraints. |
| 63 | Pad constraint file agent | 19. Timing Closure | Generates I/O-pad timing constraints. |
| 64 | Timing results analysis agent | 19. Timing Closure | Analyzes timing reports across PVT corners. |
| 65 | Timing test commands agent | 19. Timing Closure | Builds Tcl commands to probe specific paths. |
| 66 | Timing fix proposal agent | 19. Timing Closure | Proposes RTL or constraint fixes for failing paths. |
| 67 | POD agent | 20. Testing POD and software testing | Builds the POD test list from SRR and system requirements. |
| 68 | POD analysis agent | 20. Testing POD and software testing | Analyzes POD test logs; identifies issues and resolution paths. |
| 69 | FVD document agent | 20. Testing POD and software testing | Assembles the Firmware Version Document for release. |
| 70 | ATP documentation agent | 21. ATP Testing | Builds the ATP test document. |
| 71 | ATR analysis agent | 21. ATP Testing | Composes the ATR results document from test outputs. |
| 72 | ATP debug agent | 21. ATP Testing | Proposes fixes for ATP failures using current and historical results. |
| 73 | ChipScope agent | 22. Advanced Integration support | Configures Xilinx ChipScope/ILA probes. |
| 74 | SignalTap agent | 22. Advanced Integration support | Configures Intel SignalTap probes. |
| 75 | ChipScope analysis agent | 22. Advanced Integration support | Analyzes ChipScope/ILA captures. |
| 76 | SignalTap analysis agent | 22. Advanced Integration support | Analyzes SignalTap captures. |
| 77 | Debug script agent | 22. Advanced Integration support | Generates Python/MATLAB debug and log-analysis scripts. |
| 78 | Result analysis agent | 22. Advanced Integration support | Correlates system-level recordings against expected behavior. |