All five files written. Now let me actually verify the design — I'll try to install GHDL and run a loopback simulation rather than just eyeballing the RTL:All five cases pass. Last verification step — elaborate and run the actual hdlc_top (buttons, debounce, ROM loader, LEDs) to make sure the board-level integration works end to end:Everything passes. Copying the deliverables to your output folder:Complete HDLC controller for Artix-7, single 100 MHz domain — and it's not just written, it's simulation-verified with GHDL before delivery:

Design decisions worth knowing:

- Clocking: the original TxC/RxC bit clocks are replaced by a one-cycle bit_en clock enable (default 1 Mb/s via BIT_RATE_HZ generic, divider = 100). Everything registers on the 100 MHz clock — no CDC, trivial timing closure. tx_read_n and the RX strobes are single-clk pulses so the 100 MHz FIFO/host logic interacts cleanly with the slower bit cadence.

- CRC: CRC-CCITT 0x1021, all-ones preset, data LSB-first, FCS transmitted complemented MSB-first — exactly per §4.4/§6.1. I computed the receiver's good-frame residue with a software golden model: 0x1D0F, message-independent, now a constant in hdlc_package.

- RX pipeline: instead of the doc's 7-stage StatusValid alignment chain, every raw bit passes through the 8-bit R_BUFFER before being committed, so flags/aborts are detected before any of their bits could be mistaken for data — status is inherently aligned to the frame that just ended. Per the original's behavior, the two FCS bytes are delivered as data before the status byte (noted in the header comment).

- Top: BTN0 sends a 16-byte ROM message ("HELLO from HDLC!"), BTN1 aborts mid-frame + flushes the FIFO, BTN3 resets. LEDs report frame-OK / CRC_err / Abort / TX-busy. INTERNAL_LOOPBACK generic selects on-chip loopback vs. a physical jumper on PMOD JA1→JA2.

Verification results (GHDL, VHDL-2008): good frame received byte-exact (16 payload + 2 FCS + OK status), a stuffing-stress payload of 0xFF/0x7E bytes round-trips correctly, injected line-bit corruption yields CRC_err=1, mid-frame abort yields Abort=1 with the truncated frame discarded, and the link recovers with a clean frame afterward. The full board-level top (debounce, ROM loader, LEDs) also passed its own self-test. One nuance the sim surfaced: a single corrupted line bit can legitimately break destuff alignment, so Octet_err may accompany CRC_err — correct HDLC behavior, not a bug.

Compile order in Vivado: package → transmit/receive/fifo → top. The two verified testbenches (tb_hdlc_core.vhd, tb_top.vhd) exist from my verification run — say the word if you'd like them too, or a Vivado non-project Tcl build script using your SKILL flow.